LECTURE 8

NETWORKING

TCP/IP Networking Overview

What is TCP/IP Networking?

- TCP/IP networking refers to the system of communication used by computers to connect and exchange data over networks, especially the internet.

- It is the most widely used networking protocol suite in the world.

Why TCP/IP is Important

- It forms the foundation of the Internet, meaning all internet communication relies on it.

- It allows billions of devices to communicate with each other reliably.

Scale of TCP/IP (from lecture data)

- Over 2.8 billion users were online worldwide (as per older data in lecture).

- Over 1 billion devices (hosts) were connected to the internet.

- This shows that TCP/IP is designed to handle very large-scale communication.

Key Idea

- TCP/IP is not a single protocol.

- It is a collection of protocols (protocol suite) that work together to manage communication.

What TCP/IP Handles

- Sending data from one device to another

- Breaking data into smaller parts (packets)

- Routing data through networks

- Delivering data to the correct destination

What is TCP/IP?

Full Form

- TCP: Transmission Control Protocol

- IP: Internet Protocol

Basic Working Concept

- TCP/IP divides responsibilities into two main parts:

  - IP handles addressing and routing

  - TCP handles reliable delivery

Simple Understanding

- IP decides where the data should go.

- TCP ensures the data reaches correctly and in order.

Role of IP (Internet Protocol)

- IP is responsible for:

  - Assigning addresses (IP addresses)

  - Sending packets from source to destination

- It is connectionless, meaning:

  - It does not establish a connection before sending data

  - It does not guarantee delivery

Role of TCP (Transmission Control Protocol)

- TCP ensures reliable communication between devices.

- It is connection-oriented, meaning:

  - A connection is established before data transfer begins

Features of TCP

- Ensures all data is delivered

- Maintains correct order of data

- Detects and corrects errors

- Retransmits lost data if necessary

TCP/IP (Detailed Working)

Packet-Based Communication

- Data is not sent as one large block.

- It is divided into smaller units called packets.

How Data Travels in TCP/IP

- The sender divides data into packets.

- Each packet contains:

  - Data

  - Source address

  - Destination address

- Packets travel through different routes in the network.

- At the destination:

  - Packets are reassembled into the original data.

Important Concept

- All internet communication uses IP for addressing.

- TCP or UDP is used on top of IP depending on the requirement.

Protocols

Definition of Protocol

- A protocol is a set of rules that defines how data is transmitted between devices in a network.

Why Protocols are Needed

- They ensure:

  - Proper communication

  - Correct data format

  - Error handling

- Without protocols, devices would not understand each other.

Main Protocols in TCP/IP

1. Internet Protocol (IP)

- Responsible for sending packets across networks.

- It is:

  - Connectionless

  - Unreliable (no guarantee of delivery)

- Its main job is routing data to the correct destination.

2. Transmission Control Protocol (TCP)

Nature

- Connection-oriented protocol

- Provides reliable communication

Key Features

- Guarantees delivery of data

- Maintains correct order of packets

- Performs error checking and correction

Congestion Control

- TCP monitors network traffic.

- If the network is busy, it reduces the speed of data transmission.

- If the network is free, it increases speed.

- This prevents network overload.

Flow Control

- TCP ensures that the sender does not send data faster than the receiver can handle.

- It balances the speed between sender and receiver.

3. User Datagram Protocol (UDP)

Nature

- Connectionless protocol

- Faster but less reliable

Key Characteristics

- Does not guarantee delivery

- Does not ensure order of packets

- No error correction mechanism

When UDP is Used

- When speed is more important than accuracy

- Examples include:

  - Video streaming

  - Online gaming

  - Voice calls

TCP/IP Network Stack

Definition

- The TCP/IP network stack is a layered structure used to organize how data is transmitted over a network.

- Each layer performs a specific function and passes data to the next layer.

Layers of TCP/IP Stack

1. Application Layer

- This is the top layer where user interaction happens.

- It provides services directly to applications.

Functions:

- Allows applications to send and receive data

- Acts as the interface between user and network

Examples:

- Web browsers

- Email clients

Working:

- When a user sends data, this layer starts the process and passes data to the transport layer.

2. Transport Layer

- Responsible for end-to-end communication between devices.

Functions:

- Breaks data into smaller units (segments)

- Adds headers for control information

- Ensures proper delivery (if using TCP)

Protocols used:

- TCP (reliable)

- UDP (fast but unreliable)

Working:

- Receives data from application layer

- Adds a header

- Sends it to the network layer

3. Network Layer

- Responsible for addressing and routing.

Functions:

- Adds source and destination IP addresses

- Determines the best path for data

Protocol:

- IP (Internet Protocol)

Working:

- Receives segments

- Converts them into packets (datagrams)

- Sends them to the correct destination

4. Data Link Layer (Link Layer)

- Handles communication within the same network.

Functions:

- Performs error checking

- Uses MAC (hardware) addresses

- Controls data transfer between devices on same network

Working:

- Receives packets

- Converts them into frames

- Passes them to physical layer

5. Physical Layer

- The lowest layer of the stack.

Functions:

- Transmits raw data as electrical signals or bits

- Handles physical connection (cables, hardware)

Examples:

- Ethernet cables

- Fiber optics

- Wi-Fi signals

Working:

- Sends and receives actual bits over the medium

Layers + Encapsulation

What is Encapsulation?

- Encapsulation is the process of adding headers at each layer as data moves down the network stack.

How Encapsulation Works

- Data starts at the application layer as a message.

- As it moves downward:  Each layer adds its own header.

- At the receiving side:  Headers are removed in reverse order (decapsulation).

Data Transformation Across Layers

| Layer | Data Name |
| --- | --- |
| Application | Message |
| Transport | Segment |
| Network | Datagram |
| Data Link | Frame |
| Physical | Bits |

Example of Encapsulation

- Suppose an application sends 100 bytes of data:

Step-by-step addition:

- Transport layer (UDP) adds 8 bytes header

- Network layer (IP) adds 20 bytes header

- Data link layer (Ethernet) adds 18 bytes header

Final Result

- Original data becomes larger due to headers added at each layer.

Key Points to Remember

- Each layer has a specific role.

- Data is wrapped with headers at each layer.

- This wrapping process is called encapsulation.

- At the receiver, the process is reversed (decapsulation).

- Encapsulation ensures proper delivery, routing, and error handling.

Addressing in TCP/IP

Basic Idea

- Different layers in the TCP/IP model use different types of addresses.

- Each type of address serves a specific purpose in communication.

Types of Addressing by Layer

1. Application Layer Addressing

- This layer allows users to use hostnames instead of numeric addresses.

Examples:

- www.google.com

- www.example.com

Key Point

- Hostnames are human-friendly and easier to remember.

- These names are later converted into IP addresses using DNS.

2. Network Layer Addressing (IP Address)

- The network layer uses IP addresses to identify devices.

Characteristics

- Every device on a network must have a unique IP address.

- Used for routing data across networks.

Example:

- 192.168.1.1

3. Link Layer Addressing (MAC Address)

- The link layer uses MAC (Media Access Control) addresses.

Characteristics

- A MAC address is a hardware address assigned to a network interface.

- It is typically 48 bits long (for Ethernet).

Structure of MAC Address

- First 3 bytes: Manufacturer ID

- Last 3 bytes: Device-specific serial number

Example:

- 00:1A:2B:3C:4D:5E

Ports in Networking

What is a Port?

- A port identifies a specific process or service running on a device.

- It helps the system know which application should receive the data.

Key Points About Ports

- Ports are associated with applications or services.

- A single device can run multiple services using different ports.

Well-Known Ports

- A list of standard ports is stored in:

  - /etc/services

Examples:

- HTTP → Port 80

- HTTPS → Port 443

- FTP → Port 21

Privileged Ports

- Ports numbered 0 to 1024 are called privileged ports.

- These ports require administrator (root) permissions to use.

Address Types

Basic Idea

- At the IP and link layers, there are different ways to send data depending on the target.

1. Unicast

- Data is sent from one sender to one receiver.

Example

- Sending a message from your computer to a specific website server.

2. Broadcast

- Data is sent from one device to all devices in the network.

Key Feature

- All bits in the host part of the address are set to 1.

Example

- A message sent to all devices in a LAN.

3. Multicast

- Data is sent from one sender to a group of selected devices.

Key Feature

- Uses special IP address range:

  - 224 to 239 (IPv4)

Example

- Live video streaming to multiple users.

Broadcast

- Broadcast means sending data from one sender to all devices in a network.

- Every device on the network receives the data, whether it needs it or not.

Multicast

- Multicast means sending data from one sender to a selected group of devices.

- Only devices that are part of the group receive the data.

IP Addresses

What is an IP Address?

- An IP address is a unique identifier assigned to each device on a network.

- It is used to identify and locate devices for communication.

IPv4 Address Structure

- An IPv4 address consists of 4 bytes (32 bits).

- It is usually written in decimal format, separated by dots.

Example:

- 128.180.2.57

Network and Host Portions

- An IP address is divided into two parts:

  - Network portion → identifies the network

  - Host portion → identifies the device within that network

Classful Addressing (Old Method)

Originally, IP addresses were divided into classes:

| Class | First Byte Range | Format | Usage |
| --- | --- | --- | --- |
| A | 1 – 126 | N.H.H.H | Very large networks |
| B | 128 – 191 | N.N.H.H | Large organizations |
| C | 192 – 223 | N.N.N.H | Small networks |
| D | 224 – 239 | Multicast | Group communication |
| E | 240 – 255 | Experimental | Not commonly used |

Example

- IP Address: 128.180.2.57

  - Class: B (since 128–191 range)

  - Network part: 128.180

  - Host part: 2.57

Subnetting

What is Subnetting?

- Subnetting is the process of dividing a large network into smaller networks (subnets).

Why Subnetting is Needed

- Large networks are often too big to manage efficiently.

- Subnetting helps:

  - Improve network performance

  - Reduce congestion

  - Organize networks better

  - Use IP addresses efficiently

Key Concept

- Subnetting extends the network portion of an IP address into the host portion.

- This creates multiple smaller networks within a larger one.

Example from Lecture

- A Class B network like 128.180 has:

  - Total addresses: 65,534 hosts

Problem

- This is too large for practical use.

Solution

- Break it into smaller networks such as:

  - 128.180.5.*

  - 128.180.98.*

  - 128.180.14.*

Subnetting Example

Step 1: Original Network

- Network: 128.180 (Class B)

- Very large network

Step 2: Create Subnet

- Example subnet: 128.180.98

Step 3: Use Subnet Mask

- Subnet mask defines how much of the address is network vs host.

Example:

- 255.255.255.0

  - First 3 bytes → network

  - Last byte → host

CIDR Notation (Shortcut)

- Instead of writing subnet mask, we use CIDR notation.

Examples:

- 128.180.98.0/24
→ 24 bits for network

Breaking Further (Advanced Subnetting)

- Subnets can also be divided further:

Examples:

- 128.180.98.128/25

- 128.180.120.0/22

Meaning

- More bits are used for network

- Fewer bits remain for hosts

CIDR (Classless Inter-Domain Routing)

Definition

- CIDR is a method of IP addressing that removes the fixed class system (A, B, C).

- It allows networks to be defined using a prefix length, making addressing more flexible.

Key Features of CIDR

1. No Fixed Classes

- CIDR eliminates class-based addressing.

- Networks are no longer restricted to fixed sizes like Class A, B, or C.

2. Uses Prefix Length (/n)

- CIDR uses a length field to define the network portion.

Example:

- 128.180.0.0/16

  - First 16 bits → network

  - Remaining bits → host

3. Flexible Allocation

- IP addresses can be allocated based on actual need, not fixed class sizes.

- This avoids wastage of IP addresses.

4. Aggregation (Supernetting)

- Multiple smaller networks can be combined into a larger network.

Example:

- 192.200.254.0

- 192.200.255.0

Combined as:

- 192.200.254.0/23

5. Supports Efficient Routing

- Aggregated networks reduce the number of entries in routing tables.

- This improves router performance and scalability.

CIDR Notation Details

Format

- Written as:

  - IP address / n

Where:

- n = number of bits used for network

Important Formulas and Concepts

1. Number of Addresses

- Total addresses in a CIDR block:

  - 2^(32 − n)

Where n = prefix length

2. Network Address (First Address)

- Found by:

  - Performing bitwise AND between IP address and subnet mask

Formula:

- First address = (any IP) AND (network mask)

3. Last Address (Broadcast Address)

- Found by:

  - Performing OR operation with inverted mask

Formula:

- Last address = (any IP) OR [NOT (network mask)]

Advantages of CIDR

- Reduces wastage of IP addresses

- Allows flexible subnet sizes

- Supports aggregation of networks

- Reduces routing table size

- Improves overall network efficiency

NAT (Network Address Translation)

Definition

- NAT is a technique where a router modifies IP addresses and port numbers in packets.

- It allows internal (private) network devices to communicate with external networks using a public IP address.

How NAT Works

- When a packet leaves a private network:

  - The router intercepts the packet

  - Replaces:

    - Internal IP address → Public IP address

    - Internal port → External port

- When a response comes back:

  - The router uses a mapping table

  - It forwards the packet to the correct internal device

Key Features

- Maintains a mapping (translation table):

  - Ensures incoming packets are delivered to the correct internal host

- Typically:

  - Uses one public IP address

  - Uses multiple port numbers to handle many devices

- Can (in theory):

  - Map multiple hosts and ports in flexible ways

- Built into:

  - Most routers (including low-cost/home routers)

  - Operating systems like Linux

Important Understanding

- NAT allows many devices to:

  - Share a single public IP

- This helps reduce the demand for public IP addresses

Private Addresses

Basic Idea

- Private IP addresses are used inside internal networks.

- They are not routable on the internet (cannot be accessed directly from outside).

Why Private Addresses are Used

- NAT hides internal addresses, but:

  - It is not perfect

  - Some applications may include IP addresses inside data

- There can be conflicts:

  - Same IP address used internally and externally

- Solution:

  - Use private address ranges that are reserved and not used on the public internet

Private IP Address Ranges

Class A

- Range: 10.0.0.0 to 10.255.255.255

- CIDR: 10.0.0.0/8

Class B

- Range: 172.16.0.0 to 172.31.255.255

- CIDR: 172.16.0.0/12

Class C

- Range: 192.168.0.0 to 192.168.255.255

- CIDR: 192.168.0.0/16

Key Characteristics

- Not accessible from the internet

- Used in:

  - Home networks

  - Office networks

- Must use NAT to communicate with external networks

ARP (Address Resolution Protocol)

Definition

- ARP is a protocol used to find the MAC (hardware) address of a device when its IP address is known.

- It is used within a local network (LAN).

Why ARP is Needed

- Communication at different layers uses different addresses:

  - Network layer → IP address

  - Link layer → MAC address

- Problem:

  - Devices know the IP address of the destination

  - But actual delivery requires the MAC address

- Solution:

  - ARP is used to convert IP address into MAC address

How ARP Works

Step-by-Step Process

- A device wants to send data to another device on the same network.

- It knows the IP address of the destination but not the MAC address.

- It sends an ARP request (broadcast):

  - Message: “Who has this IP address?”

  - Sent to all devices in the network

- The device that owns that IP address responds with an ARP reply (unicast):

  - Sends its MAC address directly to the requester

Important Concepts

Broadcast Request

- ARP request is sent as a broadcast

- All devices receive it

Unicast Reply

- Only the correct device replies

- Reply is sent directly (unicast)

ARP Cache

- Once the MAC address is found:

  - It is stored in a table (ARP cache)

Why caching is used

- Avoids repeating ARP requests

- Improves performance

Additional Note

- ARP entries can also be learned through snooping (observing network traffic)

Key Points to Remember

- ARP maps:

  - IP address → MAC address

- Works only in local networks (LAN)

- Uses:

  - Broadcast for request

  - Unicast for reply

- Results are stored in a cache for faster future access

Simple Example

- Device A wants to send data to IP: 192.168.1.5

- It does not know MAC address

Process:

- Broadcast: “Who has 192.168.1.5?”

- Device B replies: “I have it, here is my MAC address”

- Device A stores it and sends data

Network Configuration

Definition

- Network configuration refers to the process of setting up a computer so it can communicate on a network.

Steps to Set Up a System on a Network

1. Assigning an IP Address

- Each device must have a unique IP address.

- This identifies the device on the network.

Ways to assign:

- Manually (static IP)

- Automatically using DHCP

2. Setting Hostname

- A hostname is a human-readable name given to a device.

Example:

- server1

- laptop01

Purpose

- Easier to identify devices instead of remembering IP addresses.

3. Setting Default Gateway

- The default gateway is the router’s IP address.

- It is used when sending data outside the local network.

Key Idea

- If the destination is not in the same network:

  - Data is sent to the default gateway

4. DNS Configuration

- DNS (Domain Name System) is used to convert hostnames into IP addresses.

What needs to be configured

- IP address of DNS server

Example

- When you type a website name:

  - DNS converts it into an IP address

Configuration Files (Linux-Based Systems)

Important File

- /etc/sysconfig/network-scripts/ifcfg-eth0

Contains:

- IP address

- Netmask

- Broadcast address

- Gateway

Key Points

- Network configuration is required for any system to communicate.

- It includes:

  - IP address

  - Hostname

  - Gateway

  - DNS

- Can be done manually or automatically (via DHCP)

Mapping Names to IP Addresses

Definition

- Mapping names to IP addresses means converting a hostname into its corresponding IP address.

Why It is Needed

- Humans prefer names (like websites).

- Computers communicate using IP addresses.

- So, conversion is required.

Methods of Name Resolution

1. /etc/hosts (Local File)

Description

- A simple file that stores manual mappings of hostname to IP.

Example Entry

- 127.0.0.1 localhost

- 192.168.1.10 server1

Characteristics

- Stored locally on the system

- Used when:

  - DNS is not available

  - During system startup

Advantages

- Fast (no network needed)

Disadvantages

- Not scalable for large networks

2. DNS (Domain Name System)

Definition

- DNS is a system used to convert human-readable names (hostnames) into IP addresses.

Why DNS is Needed

- Humans prefer names like:

  - www.google.com

- Computers use IP addresses like:

  - 142.250.183.14

- DNS acts as a translator between names and IP addresses.

How DNS Works

- User enters a hostname (e.g., www.example.com)

- The system sends a request to a DNS server

- DNS server looks up the IP address

- DNS server returns the IP address

- The system connects to that IP address

Advantages

- Easy to use (no need to remember IPs)

- Works globally

- Efficient and scalable

Example

- Input: www.google.com

- Output: Corresponding IP address

3. NIS (Network Information Service)

Definition

- NIS is a system used to share configuration data across multiple computers in a network.

Why NIS is Used

- In large organizations, many systems need the same data:

  - User accounts

  - Passwords

  - Hostnames

- Instead of storing this data on every machine:

  - It is stored on a central NIS server

How NIS Works

- A client system needs some information (e.g., user login)

- It sends a request to the NIS server

- The server provides the required data

- The client uses that data

Types of Data in NIS

- Usernames and passwords

- Hostnames

- Network configuration data

DHCP (Dynamic Host Configuration Protocol)

Definition

- DHCP is a protocol used to automatically assign network configuration to devices.

- Instead of manual setup, devices get settings from a DHCP server.

Key Concept: Lease

- A device does not get an IP permanently.

- It leases (borrows) network configuration for a limited time.

What DHCP Provides

A DHCP server can automatically assign:

- IP address

- Subnet mask

- Default gateway (route to internet)

- DNS server addresses

- Other configurations like:

  - Syslog servers

  - NTP (time servers)

  - Proxy servers

Why DHCP is Important

- Eliminates manual configuration

- Reduces errors

- Makes network management easier

- Used in almost all modern networks

How DHCP Works

Step-by-Step Process (High Level)

1. Client Request (Broadcast)

- When a device connects to a network:

  - It does not have an IP address

  - It sends a broadcast message:

    - “Who am I?” (request for configuration)

2. Server Response

- DHCP server replies with:

  - Available IP address

  - Network configuration details

3. Lease Assignment

- The client accepts the configuration

- The IP is leased for a specific time period

4. Lease Renewal

- When half of the lease time is over:

  - The client requests to renew the lease

- This ensures continuous connectivity

5. Server Responsibility

- DHCP server must:

  - Track all leased IP addresses

  - Maintain records even after reboot

ifconfig

Definition

- ifconfig is a command used to configure and view network interface settings.

What it is used for

- Assign or change IP address

- Set subnet mask

- Enable or disable a network interface

- View network details such as:

  - IP address

  - MAC address

  - Network status

Example

- ifconfig eth0 → shows details of interface

- ifconfig eth0 up → enables interface

- ifconfig eth0 down → disables interface

CentOS/RHEL Configuration Files

Purpose

- These files are used to store network configuration permanently in Linux systems.

Important Files

1. /etc/sysconfig/network

- Stores:

  - System hostname

  - Default gateway

2. /etc/sysconfig/network-scripts/ifcfg-*

- Used for individual network interfaces (e.g., eth0, eth1)

Contains:

- IP address

- Netmask

- Gateway

- Boot settings

3. Static Routes Files

- Used to define manual routing paths

- Helps control how data moves between networks