LECTURE 7

RAID

RAID

Definition

- RAID stands for Redundant Array of Independent Disks (originally “Inexpensive” Disks).

- It is a storage technology that combines multiple physical disks into a single logical storage unit.

Purpose

RAID is designed to:

- Improve data reliability

- Improve system performance

- Increase storage capacity

Why RAID Is Important

- A single hard disk is a single point of failure.

- If that disk fails, all stored data may be lost.

- RAID reduces this risk by distributing or duplicating data across multiple disks.

Where RAID Is Used

- Servers

- Data centers

- Enterprise storage systems

- Network Attached Storage (NAS)

- High-performance computing systems

RAID Expanded Definition & Classification)

Types of RAID Systems

There are two main types:

1. Hardware RAID

- Uses a dedicated RAID controller card.

- The controller contains built-in software to manage:

  - Disk setup

  - Input/Output (I/O) operations

  - RAID configuration

- RAID operations are handled independently of the operating system.

Advantages:

- Better performance

- Reduces CPU load on the main system

- More reliable for enterprise environments

Disadvantages:

- Expensive

- Requires dedicated hardware

2. Software RAID

- No physical RAID controller is used.

- The operating system manages RAID configuration.

- RAID processing is handled by the system CPU.

Advantages:

- Cost-effective

- Easy to implement

- Suitable for small systems

Disadvantages:

- Uses system resources

- May reduce overall system performance

Types

Hardware RAID – Detailed Operation

- Disks connect to a RAID controller.

- Controller manages:

  - Disk communication

  - Error handling

  - Parity calculations

  - Data distribution

- Appears to the operating system as a single logical drive.

Key Characteristic:

- RAID management is independent of OS failure.

Software RAID – Detailed Operation

- Managed by the operating system.

- Logical array is created via OS configuration.

- RAID level is controlled by system software.

Key Characteristic:

- RAID functionality depends on the operating system.

- If the OS fails, RAID access may be affected.

RAID (Basic Idea & Functional Goals)

Fundamental Concept

Connect multiple disks together to achieve:

1. Large Storage Capacity

- Multiple disks combine to form one large logical storage space.

2. Faster Data Access

- Data can be read or written in parallel across multiple disks.

- Parallel operations improve speed.

3. Redundant Data

- Data may be duplicated or protected with parity.

- Provides fault tolerance.

How RAID Achieves These Goals

Depending on configuration:

- Performance Increase

  - Reading/writing happens in parallel.

  - Data blocks are distributed across disks.

- Reliability Increase

  - Data duplication (mirroring)

  - Parity-based error checking

Some RAID levels focus on:

- Performance (e.g., RAID 0)

- Reliability (e.g., RAID 1)

- Balanced performance and reliability (e.g., RAID 5, RAID 6)

Important Trade-Off Factors Mentioned

RAID levels differ in:

- Redundancy level

- Error checking method

- Storage efficiency

- Performance

- Cost

This introduces the idea that:

Not all RAID configurations provide both high performance and high reliability at the same time.

Striping

Definition

Striping is a RAID technique where file data is divided into blocks and distributed across multiple disks in an array.

How Striping Works

- A file is broken into equal-sized blocks.

- Each block is written to a different disk in sequence.

Example:

- Block 0 → Disk 0

- Block 1 → Disk 1

- Block 2 → Disk 2

- Block 3 → Disk 3

This process continues in rotation.

Purpose of Striping

1. Improves Performance

- Multiple disks can be accessed simultaneously.

- Enables parallel reading and writing.

- Increases data throughput.

2. Better Bandwidth Utilization

- Workload is distributed across disks.

- Reduces single-disk bottlenecks.

Advantages

- Very fast read performance.

- Very fast write performance.

- Efficient use of total disk capacity.

Disadvantages

- No redundancy by itself.

- If one disk fails, all data in the array is lost.

- Reliability decreases as more disks are added.

Where Striping Is Used

- RAID 0

- Part of RAID 10

- Used in high-performance systems where speed is more important than data protection.

Parity

Definition

Parity is a method of error detection and data recovery used in RAID systems.

It allows reconstruction of lost data if one disk fails.

Basic Parity Concept

Parity works by:

- Counting the number of bits with value 1.

- Setting a parity bit:

  - Even number of 1s → parity bit = 0

  - Odd number of 1s → parity bit = 1

In RAID, parity is calculated using the XOR (exclusive OR) operation.

Why XOR Is Used

XOR has a special property:

If:
A ⊕ B ⊕ C = P

Then:
If A is lost →
A = B ⊕ C ⊕ P

This allows reconstruction of missing data.

Purpose of Parity in RAID

1. Fault Tolerance

- If one disk fails, lost data can be recalculated using parity information.

2. Storage Efficiency

- Uses less storage than mirroring.

- Only one disk (or equivalent space) is used for parity.

Advantages

- Provides redundancy without duplicating all data.

- More storage-efficient than RAID 1.

Disadvantages

- Parity calculations add overhead.

- Write operations are slower because parity must be updated.

- Can only recover from limited disk failures (usually one disk in RAID 5).

Where Parity Is Used

- RAID 3

- RAID 4

- RAID 5

- RAID 6 (uses double parity)

Mirroring

Definition

Mirroring is a RAID technique where identical copies of data are stored on two separate disks.

Each disk contains the exact same data.

How Mirroring Works

- When data is written:

  - It is written to Disk 1.

  - The same data is simultaneously written to Disk 2.

- When reading:

  - Data can be read from either disk.

Purpose of Mirroring

1. Data Redundancy

- If one disk fails, the other disk contains a complete copy.

- No data reconstruction is required.

2. Improved Read Performance

- System can read different parts of data from both disks simultaneously.

- Load balancing improves performance.

Advantages

- Very high reliability.

- Simple recovery process.

- Fast read performance.

- No complex parity calculations.

Disadvantages

- Expensive.

- Requires twice as many disks.

- Storage efficiency is only 50%.

- Write performance may be slightly slower (data written twice).

Where Mirroring Is Used

- RAID 1

- RAID 10 (combined with striping)

RAID Level-0 (Striping)

Definition

RAID Level-0 is the simplest RAID configuration and is often called striping.

It uses block-level striping across two or more disks.

How RAID 0 Works

- A file is divided into equal-sized blocks.

- Blocks are written across disks in alternating sequence.

Example (2 disks):

- Block 0 → Disk 0

- Block 1 → Disk 1

- Block 2 → Disk 0

- Block 3 → Disk 1

This continues in rotation.

Key Characteristics

- Minimum disks required: 2

- No parity

- No mirroring

- No redundancy

RAID 0 focuses entirely on performance and capacity, not reliability.

Performance

Read Performance

- Very high.

- Multiple disks read data simultaneously.

- Improves throughput significantly.

Write Performance

- Also very high.

- Data is written in parallel across disks.

Storage Efficiency

- 100% of total disk capacity is usable.

- If two 1TB disks are used → total usable space = 2TB.

Major Disadvantage

No Fault Tolerance

- If one disk fails, all data in the array is lost.

- Failure probability increases as more disks are added.

RAID 0 increases speed but decreases reliability.

Typical Use Cases

- Gaming systems

- Video editing (temporary projects)

- High-speed cache storage

- Situations where data loss is acceptable or backed up elsewhere

RAID Level-0 (Illustration & Data Distribution)

This visually explains how RAID 0 distributes blocks across disks.

Block Distribution Example

Given file blocks:

block 0 | block 1 | block 2 | block 3 | block 4

With 2 disks:

Disk 0 stores:

- block 0

- block 2

- block 4

Disk 1 stores:

- block 1

- block 3

This alternating pattern ensures balanced load distribution.

Sector-Level View

The diagram shows how:

- Each disk stores non-contiguous pieces of the file.

- Blocks are written in different sectors on each disk.

- Logical file appears continuous to the operating system.

Important Technical Insight

RAID 0 does not duplicate data.

Each disk contains unique portions of the file.

Therefore:

- Losing Disk 0 → blocks 0, 2, 4 are lost.

- Losing Disk 1 → blocks 1, 3 are lost.

- Missing even one block makes the entire file unusable.

Why RAID 0 Is Fast

- Parallel I/O operations

- Reduced seek time per disk

- Increased data bandwidth

- Load balancing across disks

Risk Factor

Reliability formula concept:
If each disk has probability of failure P,
RAID 0 failure probability increases with number of disks.

More disks = higher risk.

Advantages and Disadvantages

• Fast access

• If one disk fails, the entire system will no more be able to use the data on all the disks

RAID Level-1 (Data Mirroring)

Definition

RAID Level-1 is a RAID configuration that uses data mirroring, where identical copies of data are stored on two separate disks.

Each disk contains a complete and exact copy of all data.

How RAID 1 Works

- A complete file is written to Disk 0.

- The same file is simultaneously written to Disk 1.

- Both disks always contain identical information.

This means:

- Disk 0 = Full copy

- Disk 1 = Full copy

Minimum Disk Requirement

- Requires at least 2 disks.

- Can be implemented with more disks in some enterprise setups (multiple mirrors).

Redundancy Level

RAID 1 provides complete redundancy:

- If one disk fails, the system continues operating using the remaining disk.

- No data reconstruction is required (unlike parity-based RAID).

Storage Efficiency

- Only 50% of total disk capacity is usable.

- Example:

  - 2 × 1TB disks → usable space = 1TB

  - 2 × 500GB disks → usable space = 500GB

This makes RAID 1 one of the most expensive RAID implementations per usable GB.

Reliability

RAID 1 offers:

- High fault tolerance

- Protection against single disk failure

- Fast recovery (simply replace failed disk and rebuild mirror)

RAID Level-1 (Illustration & Data Layout)

Data Distribution

Given file blocks:
block 0 | block 1 | block 2 | block 3 | block 4

Both disks store:

Disk 0:

- block 0

- block 1

- block 2

- block 3

- block 4

Disk 1:

- block 0

- block 1

- block 2

- block 3

- block 4

There is no striping and no parity.

Each disk is a full replica.

What Happens During Disk Failure

If:

- Disk 0 fails → Disk 1 continues functioning normally.

- Disk 1 fails → Disk 0 continues functioning normally.

The system does not lose data.

After replacing the failed disk:

- Data from the working disk is copied to the new disk.

- This process is called rebuilding the mirror.

Key Difference from RAID 0

| RAID 0 | RAID 1 |
| --- | --- |
| Data split across disks | Data duplicated |
| No redundancy | Full redundancy |
| 100% capacity usable | 50% capacity usable |
| Very high risk | High reliability |

RAID 1 Performance

Read Performance

RAID 1 improves read performance because:

- The system can read from either disk.

- Different read requests can be handled simultaneously.

- Load balancing is possible.

In multi-user systems:

- Disk 0 may serve one request.

- Disk 1 may serve another request.

This improves overall read throughput.

Write Performance

Write performance:

- Data must be written to both disks.

- Write speed is similar to a single disk (not doubled).

- Slight overhead exists because two write operations occur.

However:

- There is no parity calculation (unlike RAID 5 or 6).

- Therefore, write penalty is minimal compared to parity-based RAID.

RAID Level-10

Definition

RAID Level-10 (also written as RAID 1+0) is a RAID configuration that combines RAID 0 (Striping) and RAID 1 (Mirroring).

It provides both:

- High performance (from striping)

- High redundancy (from mirroring)

How RAID 10 Works

RAID 10 follows a two-step structure:

Step 1: Mirroring (RAID 1)

- Disks are grouped into mirrored pairs.

- Each disk has an identical copy in its mirror partner.

Step 2: Striping (RAID 0)

- Data is striped across these mirrored pairs.

- Blocks are distributed between the mirror groups.

So the system:

- First mirrors

- Then stripes across mirrored sets

Minimum Disk Requirement

- Requires at least 4 disks

  - 2 disks per mirror set

  - At least 2 mirror sets to stripe across

Performance Characteristics

Read Performance

- Very high.

- Multiple disks can serve read requests.

- Combines read benefits of RAID 0 and RAID 1.

Write Performance

- High performance due to striping.

- No parity calculations required.

- Faster than RAID 5 and RAID 6 for write-heavy workloads.

Redundancy & Fault Tolerance

- Can tolerate multiple disk failures, as long as:

  - Both disks in the same mirrored pair do not fail.

Example (4 disks):

- Disk 1 fails → OK

- Disk 3 fails → OK

- But if Disk 1 and its mirror fail → Data loss

Storage Efficiency

- 50% usable storage (same as RAID 1).

Example:

- 4 × 1TB disks → usable space = 2TB

- 6 × 2TB disks → usable space = 6TB

Half of total storage is used for redundancy.

Cost

- Considered the most expensive RAID configuration.

- Requires double the storage for redundancy.

- But provides best balance of performance and reliability.

RAID 10 (Diagram Explanation)

Understanding the Diagram

Two possible conceptual views are shown:

RAID 0+1 (Mirror of Stripes)

- Data is first striped.

- Then the striped sets are mirrored.

- Less fault tolerant than RAID 1+0.

RAID 1+0 (Stripe of Mirrors) – True RAID 10

- Disks are first mirrored.

- Then data is striped across mirrored sets.

- More fault tolerant.

- This is the standard RAID 10 implementation.

RAID 0+1 vs RAID 1+0 (Important Distinction)

| RAID 0+1 | RAID 1+0 |
| --- | --- |
| Mirror of stripes | Stripe of mirrors |
| Less fault tolerant | More fault tolerant |
| Entire stripe group may fail | Only affected mirror pair fails |
| Rarely used today | Common enterprise solution |

RAID 10 refers specifically to RAID 1+0, not RAID 0+1.

Why RAID 10 Is High Performance

- No parity calculations

- Parallel read/write operations

- Reduced rebuild stress compared to RAID 5

- Fast recovery (mirror rebuild only)

Where RAID 10 Is Used

- Database servers

- High-transaction systems

- Virtual machines

- Enterprise applications

- Systems requiring both speed and reliability

RAID 2 (Bit-Level Striping with Dedicated Hamming Code Parity)

Definition

RAID 2 uses:

- Bit-level striping

- Dedicated error correction using Hamming Code

- Separate disks to store error-correcting information

How RAID 2 Works

- Data is split at the bit level (not block or byte).

- Each bit is written to a different disk.

- Additional disks store Hamming Code parity for error correction.

Example:

- Disk 1 → Bit 1

- Disk 2 → Bit 2

- Disk 3 → Bit 3

- Extra disks → Error correction codes

Key Characteristics

- Requires multiple disks for both data and error correction.

- Uses complex Hamming Code instead of simple XOR parity.

- Can detect and correct errors automatically.

Why RAID 2 Is Rarely Used

- Modern disks already have built-in error correction.

- Requires high synchronization between disks.

- Very expensive and complex.

- Inefficient compared to RAID 5 and RAID 6.

As a result:

RAID 2 is considered obsolete in modern systems.

RAID 3 (Byte-Level Striping with Dedicated Parity)

Definition

RAID 3 uses:

- Byte-level striping

- One dedicated parity disk

Unlike RAID 5:

- Parity is stored on a single separate disk.

- Parity is not distributed across all disks.

How RAID 3 Works (According to the Diagram)

In the image, there are 4 disks:

- Disk 0

- Disk 1

- Disk 2

- Disk 3 (Parity Disk)

Data Distribution

Look at the first row in the diagram:

- Disk 0 → A1

- Disk 1 → A2

- Disk 2 → A3

- Disk 3 → Ap(1–3)

This means:

- A1, A2, A3 are pieces of the same data stripe.

- Ap(1–3) is the parity calculated from A1, A2, and A3.

Parity is calculated using XOR:

Ap(1–3) = A1 ⊕ A2 ⊕ A3

Next Stripe

Second row:

- Disk 0 → A4

- Disk 1 → A5

- Disk 2 → A6

- Disk 3 → Ap(4–6)

Again:

- A4, A5, A6 are split across the three data disks.

- Parity for them is stored on Disk 3.

Same Pattern Continues (Group B)

For B data:

- Disk 0 → B1

- Disk 1 → B2

- Disk 2 → B3

- Disk 3 → Bp(1–3)

And then:

- Disk 0 → B4

- Disk 1 → B5

- Disk 2 → B6

- Disk 3 → Bp(4–6)

Each row represents one stripe group.

What Happens If a Disk Fails?

Suppose Disk 1 fails (A2, A5, B2, B5 lost).

To recover A2:

A2 = A1 ⊕ A3 ⊕ Ap(1–3)

The system uses:

- Remaining data disks

- Parity disk

to reconstruct the missing data.

Minimum Disk Requirement

- At least 3 disks

  - 2 data disks

  - 1 parity disk

Performance Characteristics

Sequential Access Performance

- Very good.

- All disks work together to read/write large continuous data.

- Ideal for large file transfers.

Small or Random Access Performance

- Poor.

- Every I/O operation must involve all disks.

- Parity disk becomes a bottleneck.

Disadvantages

- Dedicated parity disk causes write bottleneck.

- Not efficient for multiple small requests.

- Not suitable for multi-user environments.

- Largely replaced by RAID 5.

Fault Tolerance

- Can survive failure of one disk.

- If parity disk fails, system still functions but without redundancy until replaced.

RAID 4 (Block Level Striping with Dedicated Parity)

How It Works (Based on the Diagram)

In the screenshot, there are four disks:

- Disk 0

- Disk 1

- Disk 2

- Disk 3 (Parity Disk)

1. Block-Level Striping

RAID 4 divides data into full blocks (not bytes).

From the diagram:

First stripe:

- Disk 0 → A1

- Disk 1 → A2

- Disk 2 → A3

- Disk 3 → Ap

Here:

- A1, A2, A3 are complete data blocks.

- Ap is the parity block calculated from A1, A2, and A3.

Parity formula:
Ap = A1 ⊕ A2 ⊕ A3

Second stripe:

- Disk 0 → B1

- Disk 1 → B2

- Disk 2 → B3

- Disk 3 → Bp

Bp = B1 ⊕ B2 ⊕ B3

The same pattern continues for C and D blocks.

Each row represents one block stripe group.

2. Dedicated Parity Disk

Disk 3 stores only parity blocks:

- Ap

- Bp

- Cp

- Dp

All parity information is centralized on this single disk.

3. Data Recovery

If Disk 1 fails (A2, B2, C2, D2 lost):

Example recovery:
A2 = A1 ⊕ A3 ⊕ Ap

The system uses:

- Remaining data blocks

- Parity block

RAID 4 can tolerate one disk failure.

4. Performance Characteristics

Read Performance:

- Good for small reads.

- Since striping is at block level, a single block can be read from one disk.

Write Performance:

- Slower than expected.

- Every write must update:

  - The data block

  - The parity block on Disk 3

This makes Disk 3 a bottleneck.

RAID 5 (Block Level Striping with Distributed Parity)

Definition

RAID 5 uses:

- Block-level striping

- Distributed parity

It stripes both data and parity information, adds redundancy, and improves read performance while maintaining good storage efficiency.

How RAID 5 Works

1. Block-Level Striping

File data is divided into blocks and distributed across all disks in the array.

Example (4 disks):

Stripe 1:

- Disk 0 → A1

- Disk 1 → A2

- Disk 2 → A3

- Disk 3 → Ap (parity)

Stripe 2:

- Disk 0 → B1

- Disk 1 → B2

- Disk 2 → Bp (parity)

- Disk 3 → B3

Stripe 3:

- Disk 0 → C1

- Disk 1 → Cp (parity)

- Disk 2 → C2

- Disk 3 → C3

Parity blocks rotate across disks.
There is no single dedicated parity disk.

No Single Check Disk

It is stated that:

- There is no longer a single check disk.

- There is no write bottleneck caused by one parity disk.

Since parity is distributed, write operations can occur in parallel across disks.

Fault Tolerance

- Provides fault tolerance using parity.

- Data and parity information are distributed over all disks.

- If one disk fails, data can be reconstructed.

Example:
If A2 is lost:

A2 = A1 ⊕ A3 ⊕ Ap

RAID 5 can tolerate failure of one disk only.

Storage Efficiency

If there are N drives in the array (minimum 3 required):

Usable storage = N − 1 drives

Examples:

- 3 drives → 2 drives usable

- 4 drives → 3 drives usable

Space efficiency is at least 67%, which is higher than mirroring (maximum 50%).

Performance

- Radically improves performance of multiple writes because writes can be done in parallel.

- Slightly improves read performance because there is one more disk available for reading.

- Write performance is slower than RAID 0 and RAID 10 due to parity calculations, but better than RAID 4 because parity is distributed.

RAID 6 (Block Level Striping with Double Distributed Parity)

Definition

RAID 6 is similar to RAID 5 but uses two parity blocks per stripe instead of one.

It provides:

- Block-level striping

- Double distributed parity

- Higher fault tolerance than RAID 5

How RAID 6 Works

1. Block-Level Striping

Data is divided into blocks and distributed across disks, just like RAID 5.

Example (5 disks):

Stripe 1:

- Disk 0 → 1a

- Disk 1 → 1b

- Disk 2 → 1c

- Disk 3 → 1p (Parity 1)

- Disk 4 → 1q (Parity 2)

Stripe 2:

- Disk 0 → 2a

- Disk 1 → 2b

- Disk 2 → 2p

- Disk 3 → 2q

- Disk 4 → 2c

Parity blocks rotate across disks, meaning no single disk is dedicated only to parity.

2. Double Distributed Parity

Each stripe has:

- Two independent parity calculations (often called P and Q).

This means:

Parity 1 = XOR-based parity
Parity 2 = Second independent parity calculation

Because there are two parity values, the system can recover from two disk failures.

3. Fault Tolerance

RAID 6 can withstand:

- Complete failure of two drives without losing data.

If:

- One disk fails → Data rebuilt using parity.

- Two disks fail → Data still rebuilt using double parity.

If three disks fail → Data loss occurs.

4. Minimum Disk Requirement

- Requires at least 4 disks.

5. Storage Efficiency

If there are N disks:

Usable storage = N − 2 disks

Example:

- 4 disks → 2 usable

- 5 disks → 3 usable

- 6 disks → 4 usable

Storage efficiency is lower than RAID 5 but higher than RAID 1 for larger arrays.

6. Performance Characteristics

Read Performance:

- Good.

- Similar to RAID 5.

Write Performance:

- Slower than RAID 5.

- Two parity calculations must be performed.

- Higher write penalty.