LECTURE 5

Workstation & Servers

Workstation vs Server

Basic Idea

A workstation is a computer used by one person to do personal or office work such as typing documents, coding, or designing. All of its power and resources are used by that single user. If a workstation stops working, only that one user is affected.

A server is a computer system that provides services to many users or computers through a network. These services can include storing files, running websites, managing databases, or controlling user logins. Because many users depend on a server, it must work continuously.

Dependency and Failure Impact

The main difference between a workstation and a server is how many people depend on it. A workstation failure affects one person only, but a server failure can affect many users at the same time. Due to this, servers are designed to be more reliable and are managed more carefully.

Design Focus

Workstations focus on user comfort and performance, while servers focus on uptime, reliability, and security. Servers usually have better hardware, backup systems, and stronger security. This is why servers need a different and stricter management approach than workstations.

Managing Servers

Handling Many Users

Servers are made to handle many users at once. Managing servers means making sure that system resources like CPU, memory, storage, and network are used properly so that services remain fast and available for everyone.

Reliability and Uptime

Servers are expected to run all the time (24/7). Administrators must avoid unnecessary changes and perform updates carefully to prevent downtime. Keeping the server running is one of the most important goals of server management.

Security

Servers are usually connected to networks and sometimes the internet, which makes them more vulnerable to attacks. Therefore, servers need strong security, such as limited user access, strong passwords, and regular updates. Security on servers is stricter than on workstations because more users are affected if something goes wrong.

Cost and Life Span

Servers are more expensive than workstations, but they are built to last longer. Since many users share one server, the cost is divided among them. Also, server downtime can be very costly, so investing in good server hardware makes sense.

Operating System Setup

Servers usually run a simple and minimal operating system with only necessary software installed. This helps improve performance and reduces security risks. Unlike workstations, servers do not run unnecessary applications.

Deployment Location

Servers are placed in data centers or server rooms, not on office desks. These locations provide proper cooling, power backup, and physical security, which help keep servers running safely and smoothly.

Maintenance and Backups

Servers need regular maintenance. Many organizations use maintenance contracts to quickly fix hardware problems. Servers also use regular backups to protect important data, because data loss on a server can affect many users.

Remote Management

Servers are managed remotely using special tools. This allows administrators to check system health, fix problems, and restart servers without being physically present. Remote management is very important when managing many servers.

Server Hardware

Key Principle

The most important rule in server design is to use hardware that is made for server workloads. Servers perform tasks that are very different from normal desktop computers, so using the right hardware is essential for stable and reliable operation.

Purpose of Server Hardware

Server hardware is built to run continuously (24/7) without shutting down. Unlike desktop computers, servers are expected to work all the time and handle requests from many users. Servers also perform heavy input/output (I/O) operations, such as frequent disk access and network communication. In addition, server hardware is designed to support scalability, meaning it can be upgraded or expanded as the workload increases.

Problems with Using Desktop Hardware

Using desktop hardware for servers can cause many issues. Desktop systems are not designed for continuous use or heavy workloads, which increases the chance of hardware failure. This can lead to more downtime, where services become unavailable to users. Over time, this also increases maintenance and repair costs, making desktop hardware unsuitable for server environments.

Server Hardware – Extensibility & CPU Performance

Extensibility

Servers are designed to grow over time instead of being replaced completely. This is known as extensibility. Server hardware usually provides extra CPU sockets, more RAM slots, and additional PCIe lanes so that new components can be added easily. Servers also support advanced features such as clustering and load balancing, which allow multiple servers to work together and share the workload. This design makes servers flexible and future-proof.

CPU Performance

When choosing CPUs for servers, the goal is not just speed but long-term usefulness. Faster CPUs help delay obsolescence, meaning the server remains useful for a longer period. Many servers support multiple CPUs, which improves parallel processing by allowing many tasks to run at the same time. This is especially important for virtualization and concurrent workloads, where multiple users or applications are running simultaneously.

Server Hardware – High-Performance I/O & Upgrade Options

Importance of I/O Performance

In servers, input/output (I/O) performance is often more important than CPU speed. This is because servers usually handle many requests at the same time, which puts heavy load on disk access and network communication. Even if a server has a fast CPU, slow disks or a slow network can become a bottleneck and reduce overall performance.

I/O-Focused Server Hardware

To handle heavy I/O workloads, servers use special hardware such as RAID storage systems to improve data reliability and speed. Modern servers often use SSD or NVMe drives instead of traditional hard disks for faster data access. Servers also include multiple network interface cards (NICs) and high-bandwidth buses to support fast and reliable network communication for many users at once.

Upgrade Strategy

Servers are designed to be upgraded gradually instead of being replaced completely. As workload increases, administrators can add more RAM, replace existing CPUs with faster ones, or add additional storage disks. This incremental upgrade approach increases the server’s life span and reduces long-term costs.

Server Hardware – Rack-Mountable Design

Data Center Efficiency

Servers are usually rack-mounted in data centers. Rack mounting helps save physical space, improves airflow for cooling, and makes cable management easier. It also allows organizations to scale their infrastructure by adding more servers to the rack without wasting space.

Technical Design Features

Rack-mountable servers follow strict design rules. They have a rectangular shape so they fit properly in standard racks. Storage drives are placed at the front for easy replacement, while cables and power connections are at the rear for better organization. Servers are designed so that no side access is required, which makes maintenance easier when servers are tightly packed in racks.

Server Hardware – Maintenance, No Side Access & High Availability

No Side-Access Requirement

Rack-mounted servers are designed so that no side access is needed. This is important because servers in racks are placed very close to each other, and their sides are usually not reachable. For easy maintenance, all important components must be accessible from the front or back of the server.

Hard drives are placed at the front, while power cables and network connections are placed at the back. This design allows administrators to repair or replace parts without removing the server from the rack, saving time and effort.

Maintenance-Friendly Design

Servers should be easy to repair while they are still running or mounted in a rack. Tasks such as replacing disks or checking connections should not require moving the server. This reduces downtime and makes server maintenance faster and more efficient, especially in large data centers.

High Availability (HA)

High availability means that a server should continue working even if one component fails. To achieve this, servers use special hardware features such as dual power supplies, so if one power supply fails, the other keeps the server running. Servers also use RAID disks, which protect data if a hard drive fails.

In addition, servers often have multiple network connections, so network communication continues even if one connection goes down. Many server components are hot-swappable, meaning they can be replaced while the server is still running.
The main goal of high availability is to avoid a single point of failure.

Server Hardware – Management Options

Servers should support remote management so administrators can manage and fix them without physical access to the machine. This is especially useful when the operating system is not responding or the server is completely down. Servers often provide serial port access, which allows administrators to connect directly to the system at a very low level and view system messages, BIOS output, or boot process information. Using remote management and serial access, administrators can diagnose hardware or system problems, reboot the server, and restore it to normal operation, even when normal network access is unavailable.

Do Servers Really Cost More?

Overview

Most hardware vendors offer their systems in three main product lines: Home, Business, and Server. Each product line differs in terms of cost, reliability, component stability, and long-term value.

Home Systems

Home systems have the lowest purchase price, making them attractive initially. However, their components change frequently, and they are not designed for long-term or continuous use. This frequent change makes maintenance difficult and increases long-term cost.

Business Systems

Business systems are more reliable than home systems. They are designed to have a longer life span and fewer component changes, which improves stability and makes them easier to support. However, they still offer limited performance and serviceability compared to server systems.

Server Systems

Server systems provide the lowest cost per performance metric because they are designed to serve many users efficiently. Their components and overall design are easier to service, which reduces downtime and maintenance effort. Although servers have a higher initial cost, they offer better long-term value.

Maintenance Contracts & Spare Parts

Hardware Failure Is Inevitable

In computer systems, hardware failure is unavoidable. No matter how reliable a machine is, all components eventually wear out or fail. Because servers provide important services, system administrators do not assume that failures can be completely avoided. Instead, they plan in advance so that services can be restored quickly when failures occur.

Vendor Maintenance Contracts

Hardware vendors offer a variety of maintenance and service contracts to handle system failures. These contracts usually include on-site support, where a technician visits the location to repair or replace faulty components. The response time depends on the contract and may be 4-hour, 12-hour, or next-day service. Faster response times are more expensive but reduce downtime.

In customer-purchased spare arrangements, the organization keeps its own approved spare parts on-site, the vendor uses those spares to repair the organization’s servers when needed and then replaces the used spares under the maintenance contract.

Choosing the Right Maintenance Contract

Selecting a maintenance contract depends on the importance and role of the system.

Non-Critical Hosts

Non-critical hosts are systems where downtime does not cause serious problems. For such systems, a next-day or two-day maintenance response is usually enough. In some cases, the organization may not need a maintenance contract at all, since temporary downtime is acceptable.

Large Groups of Similar Hosts

When an organization manages many identical or very similar systems, a spares-based approach is effective. Instead of paying for fast service contracts, the organization keeps spare machines or spare parts on-site. When a failure occurs, the faulty system or part is immediately replaced, reducing downtime.

Controlled Hardware Model

A controlled hardware model means an organization uses only a few standard types of hardware instead of many different ones. This makes systems easier to manage and repair. It also reduces the number of spare parts needed, which lowers maintenance cost and downtime.

Critical Hosts

Critical hosts are systems where downtime can seriously affect users or business operations. For these systems, administrators should keep failure-prone and easily replaceable parts such as power supplies and hard drives in stock. For other components, a same-day maintenance contract is usually required to ensure quick recovery and minimal downtime.

Large Installations with Many Models

In large organizations that use a wide variety of systems from the same vendor, it may be more efficient to have a vendor-provided on-site technician. This allows faster repairs, better coordination, and reduced downtime across many systems.